EXPERIMENT 1A

Name:- B Dharanidaran

Reg. No. :- 192412673

Program Code:

clc;

clear;

close all;

%% WR-90 Rectangular Waveguide Parameters

a = 22.86e-3;       % Width in meters

b = 10.16e-3;       % Height in meters

L = 100e-3;         % Length in meters

f = 10e9;           % Operating frequency = 10 GHz

c = 3e8;            % Speed of light (m/s)

mu0 = 4*pi*1e-7;    % Permeability of free space

eps0 = 8.854e-12;   % Permittivity of free space

%% Theoretical Cutoff Frequency for TE10 Mode

fc = c/(2*a);

fprintf('THEORETICAL RESULTS\n');

fprintf('Cutoff frequency fc = %.4f GHz\n', fc/1e9);

%% Propagation Constant

if f > fc

beta = (2*pi*f/c)*sqrt(1-(fc/f)^2);

fprintf('Propagation constant beta = %.4f rad/m\n', beta);

else

beta = 0;

fprintf('Wave does not propagate at this frequency.\n');

end

%% Waveguide Coordinates

Nx = 100;

Nz = 100;

x = linspace(0,a,Nx);

z = linspace(0,L,Nz);

[X,Z] = meshgrid(x,z);

%% TE10 Electric Field Distribution

% Dominant electric field component is Ey

Ey = sin(pi*X/a).*exp(-1j*beta*Z);

figure;

surf(X*1000,Z*1000,abs(Ey));

shading interp;

colorbar;

xlabel('Width x (mm)');

ylabel('Length z (mm)');

zlabel('|Ey| (V/m, normalized)');

title('TE10 Mode - Electric Field Distribution');

view(45,30);

%% TE10 Magnetic Field Distribution

% Hx component for TE10 mode

omega = 2*pi*f;

Hx = -(beta/(omega*mu0))*Ey;

figure;

surf(X*1000,Z*1000,abs(Hx));

shading interp;

colorbar;

xlabel('Width x (mm)');

ylabel('Length z (mm)');

zlabel('|Hx| (A/m, normalized)');

title('TE10 Mode - Magnetic Field Distribution');

view(45,30);

%% Electric Field Cross-Section

figure;

plot(x*1000, sin(pi*x/a),'LineWidth',2);

grid on;

xlabel('Width x (mm)');

ylabel('Normalized Electric Field');

title('TE10 Electric Field Across Waveguide Width');

%% Summary

fprintf('\nSIMULATION SUMMARY\n');

fprintf('Operating frequency = %.2f GHz\n',f/1e9);

fprintf('Cutoff frequency = %.4f GHz\n',fc/1e9);

fprintf('Propagation constant beta = %.4f rad/m\n',beta);

fprintf('TE10 mode propagates because f > fc.\n');

OUTPUT